Drone Control System + Image Processing

Combined Project Documentation

1. Product Requirements Document (PRD)

Project Overview

This project is a quadcopter drone simulation and control system developed using Python and PyBullet. The goal of the system is to simulate realistic drone movement, implement manual drone control, and build a foundation for future image processing applications, namely, a homography application and image warping.

The project combines:

- Drone physics simulation

- Real-time user control

- 4-motor thrust management

- Torque-based movement

- Camera/image processing concepts

- Modular drone architecture

Problem Statement

Developing and testing drone control systems on real hardware can be expensive, risky, and difficult to debug. A simulation environment allows safer testing and easier debugging. Besides that, testing image processing software requires sample pictures with metadata, which can be hard to acquire without a real drone system. This project solves these two problems by creating a controllable simulated drone platform that mimics quadcopter behavior inside a physics engine and captures images with metadata for image processing applications.

Target Users

Primary Users

- Robotics students

- Computer vision students

- Developers learning drone control

- Researchers prototyping autonomous systems

Secondary Users

- Hobbyists experimenting with drone simulation

- Developers building AI navigation systems

Main Features

Core Drone Features

- 4-motor quadcopter system

- Real-time thrust control

- Pitch control

- Roll control

- Yaw control (on joystick version)

- Hover adjustment

- Keyboard or joystick-based flight input

Simulation Features

- PyBullet physics integration

- Gravity simulation

- Real-time force application

- 3D drone environment

Image Processing Foundation

- Camera positioning system

- Image capture pipeline

- Homography and projection experimentation: forward/inverse mapping

Architecture Features

- Object-oriented drone model

- Modular motor classes

- Expandable control system

- Separation between physics, control, and image capture/processing logic

2. Functional Specification (FSPEC)

System Behavior

The system simulates a quadcopter drone inside a PyBullet environment. The user controls the drone through keyboard/joystick commands that modify motor thrust values.

The simulation loop:

- Reads user input

- Updates motor thrust

- Applies forces to the drone body

- Advances physics simulation

- Updates drone position and orientation

- Captures images if enabled

The image processing unit takes input data (the taken pictures + metadata of drone position/orientation), and computes the homography matrix. After applying the homography, the warped results are stored.

Functional Components

2.1 Motor System

Each motor has:

- Position relative to drone body

- On/off state

- Thrust value

Motor behavior:

- Motors must be activated before thrust is applied

- Negative thrust is prevented

- Motors independently contribute to total drone movement

2.2 Drone System

The drone contains:

- Four motors

- Grouped motor regions:

  - Front motors

  - Rear motors

  - Left motors

  - Right motors

The drone supports:

- Hover control

- Pitch movement

- Roll movement

- Yaw movement (only in joystick version)

- Force balancing

Implementation structure:

2.3 Control Inputs

Joystick controls are taken from the configuration described in the README.md file for consistency.

Hover Controls

- Q or left joystick down → Increase vertical thrust

- E or left joystick up→ Decrease vertical thrust

- R → Reset thrust and position

- O → Turn on all motors (input in terminal, not simulation window)

Pitch Controls

- W or right joystick up → Pitch forward

- S or right joystick down → Pitch backward

Pitch is achieved by increasing thrust on one side while decreasing thrust on the opposite side.

Roll Controls

- A or right joystick left → Roll left

- D or right joystick right → Roll right

Roll is achieved by creating thrust imbalance between left and right motors.

2.4 Simulation Flow

Startup

- Initialize PyBullet

- Load plane and drone URDF

- Create drone object

- Wait for motor activation

Runtime Loop

- Detect pressed keys

- Process movement commands

- Update thrust values

- Apply forces to drone body

- Advance simulation step

2.5 Physics Interaction

The drone uses:

- External force application

- Real-time simulation stepping

- Quaternion orientation tracking

- Rotation matrix computation

Each motor applies an upward force at its physical position relative to the drone body.

3. Development Specification (DEVSPEC)

Technologies Used

Programming Language

- Python

Main Libraries

- NumPy

- PyBullet

- OpenCV

Simulation Environment

- PyBullet physics engine

System Architecture

The project uses an object-oriented architecture.

Main Modules

Motor Class

Responsible for:

- Individual motor state

- Thrust management

- Motor position storage

Drone Class

Responsible for:

- Managing all motors

- Flight behavior

- Torque balancing

- Movement logic

Control Loop

Responsible for:

- Keyboard input

- Physics updates

- Applying external forces

- Running simulation timing

Image capturing pipeline

Responsible for:

- Configuring camera/position and intrinsics

- Capturing images from the simulation environment (which can be tailored for testing needs by adding 3D objects)

- Retrieving drone position and orientation

- Storing the picture and its metadata

Key Design Decisions

3.1 Modular Motor Design

Each motor is implemented as a separate object rather than storing all thrust values in arrays only.

Advantages:

- Easier debugging

- Better scalability

- Cleaner physics management

- Future support for damaged/failing motors

- Enables new motor features to be added in the future: power, rpm, etc.

3.2 Torque-Based Movement

Instead of directly rotating the drone, movement is generated physically through motor thrust differences.

Advantages:

- More realistic flight behavior

- Physics-driven movement

- Easier future PID integration

The project already outlines a future architecture:

# input → desired state → PID → mixer → motors

3.3 Real-Time Physics Simulation

The system uses:

- Fixed timestep simulation

- Real gravity

- External force application

Simulation timestep:

dt = 1/240, which matches PyBullet’s 240Hz.

3.4 Expandability

The project was designed to later support:

- PID controllers

- Autonomous stabilization

- Object tracking

- Image-based navigation

- Real drone hardware integration

4. Testing

Testing Approach

Testing of control system was performed manually inside the PyBullet simulation environment.

Testing of image processing is performed by capturing the pictures, applying the homography and visually comparing the results.

Test Cases

4.1 Drone flight

Objective

Ensure the drone flies according to the implemented logic.

Procedure

- Start simulation

- Activate motors using O

- Enable hover using the corresponding input

- Control orientation

Expected Result

- The drone should fly according to a simplified, realistic model of flight.

Result

Passed

4.2 Image capturing test

Objective

Verify that the images are taken from the correct position and orientation, and that they are stored properly with the corresponding metadata.

Procedure

- Enable the camera

- Configure desired camera position and specifications

Expected Result

- Images of the plane are stored in the dataset folder

Result

Passed

4.3 Image processing Test

Objective

Verify that the images appear from vertical BEV after application of the image processing unit.

Procedure

- Apply the image processing unit on the raw dataset.

Expected Result

- Images appear warped and from vertical BEV

Result

Passed

5. Benchmarking / Performance Evaluation

Performance Evaluation

Since this is an educational simulation project, benchmarking focused on responsiveness, modularity, and simulation realism rather than industrial-grade flight accuracy.

Evaluation Criteria

Responsiveness

The drone reacts immediately to the user's keyboard input with minimal latency.

Physics Accuracy

Movement is generated through realistic thrust application rather than artificial transformations.

Stability

The simulation maintains stable execution at:

240 Hz physics timestep

Current Limitations

- No PID stabilization yet

- No aerodynamic drag model

- No autonomous navigation

- No obstacle avoidance

- Simplified quadcopter dynamics

- Simple homography application, no stitching

Future Improvements

Control Improvements

- PID stabilization

- Altitude hold

- Autonomous hover

Image Processing

- Real-time camera feed

- Object detection

- SLAM integration

- Terrain mapping

Simulation Improvements

- Better drone physics

- Wind simulation

- Collision handling

- Battery simulation

Conclusion

This project successfully implements a modular quadcopter simulation and image processing system using Python and PyBullet. The system demonstrates:

- Real-time drone control

- Physics-based flight behavior

- Modular software architecture

- Image capturing

- Homography application for BEV images

- Expandable simulation design

The project establishes a strong foundation for future work in:

- Autonomous drones

- Computer vision

- Robotics research

- Flight control systems